SLIP 8

Aim

To perform raster analysis using population density grids and surface elevation data in QGIS.

Part 1: Geo-reference Maharashtra Map

Step 1: Open Shapefile

- Open QGIS.

- Add Ind_adm0.shp file.

Step 2: Open Georeferencer

- Go to Raster → Georeferencer.

- Load Maharashtra map image.

Step 3: Add Ground Control Points

- Select points on map image.

- Match them with correct location on Ind_adm0.shp layer.

Step 4: Start Georeferencing

- Select transformation type Linear.

- Click Start Georeferencing.

Part 2: Population Density Analysis

Step 1: Import Raster Files

- Import raster layers:

  - Population Density Grid 1990

  - Population Density Grid 2000

Step 2: Calculate Population Difference

- Go to Raster → Raster Calculator.

- Use expression:

Population_2000 - Population_1990

- Save output raster.

Step 3: Classify Population Change

- Open Layer Properties → Symbology.

- Select Graduated Color.

- Use different colors to represent population increase or decrease.

Part 3: Hillshade Analysis

Step 1: Import Elevation Raster

- Add raster file
10n060e_20101117_gmted_mea300.tif.

Step 2: Extract Area

- Go to Raster → Extraction → Clip Raster by Extent.

- Select North India region.

Step 3: Perform Hillshade

- Go to Raster → Analysis → Hillshade.

- Select elevation raster.

- Generate hillshade output.

Result

Population density change and hillshade analysis of elevation data were successfully performed using QGIS.

✅ Ab Slip 9 & 10 aane wale hain jo thode important practical topics cover karte hain:

- Terrain Analysis

- Natural Calamity CSV analysis

- Attribute table queries

Ye exam me bahut frequently aate hain.

Agar bolo to main Slip 9-10 bhi ready answer format me bana deta hoon (aise hi likhne layak).