Ab main Slip 7 aur Slip 8 ka clear exam-ready answer de raha hoon. Tum ise practical exam ya journal me directly likh sakte ho. Steps QGIS workflow ke according hain.

SLIP 7

Aim

To create a residential map, add map layout elements and perform geo-referencing of Girgaum Chowpatty Beach using QGIS.

Part 1: Create Map of Residential Area

Step 1: Open QGIS

- Start QGIS software.

- Create New Project.

- Set CRS as WGS84 (EPSG:4326).

Step 2: Create Area Layer

- Go to Layer → Create Layer → New Shapefile Layer.

- Geometry type: Polygon.

- Add fields:

  - id (Integer)

  - name (Text)

- Save as Area.shp.

- Digitize 4 areas.

Step 3: Create Garden Layer

- Create Polygon layer called Garden.shp.

- Fields:

  - id

  - name

- Digitize 4 gardens.

Step 4: Create Railway Track Layer

- Create Line layer named RailwayTrack.shp.

- Fields:

  - id

  - name

  - line

- Digitize 2 railway tracks.

Step 5: Create Railway Stations

- Create Point layer named Station.shp.

- Fields:

  - id

  - name

  - line

- Add 5 railway stations.

Step 6: Create Restaurants

- Create Point layer called Restaurant.shp.

- Fields:

  - id

  - name

- Add 5 restaurants.

- Ensure 2 restaurants are inside gardens.

Part 2: Create Map Layout

Step 1: Open Print Layout

- Go to Project → New Print Layout.

- Enter layout name.

Step 2: Add Map

- Click Add Map Tool.

- Drag area to display the map.

Step 3: Add North Arrow

- Click Add Item → Add North Arrow.

- Place it on the map layout.

Step 4: Add Legend

- Click Add Item → Add Legend.

- Legend automatically displays map layers.

Step 5: Add Scale

- Click Add Item → Add Scale Bar.

- Adjust units to meters or kilometers.

Step 6: Add Map Title

- Click Add Label Tool.

- Enter title:
“Residential Area Map”.

Step 7: Metro Station Symbol

- Open Station Layer Properties.

- Change one station symbol to Metro symbol.

Step 8: Export Map

- Click Layout → Export as Image.

- Choose JPEG format.

- Set Resolution = 1200 dpi.

- Save the map.

Part 3: Geo-reference Girgaum Chowpatty

Step 1: Open OpenStreetMap

- Add XYZ Tiles → OpenStreetMap.

Step 2: Search Location

- Use search tool.

- Type Girgaum Chowpatty Beach.

Step 3: Open Georeferencer

- Go to Raster → Georeferencer.

- Open the aerial image of Girgaum Chowpatty.

Step 4: Add Control Points

- Use Add Point Tool.

- Select matching points on image and map.

- Add 4-6 Ground Control Points.

Step 5: Start Georeferencing

- Choose transformation type Linear.

- Click Start Georeferencing.

Result

The residential map was created successfully and the aerial image of Girgaum Chowpatty was geo-referenced using QGIS.

SLIP 8

Aim

To perform raster analysis using population density grids and surface elevation data in QGIS.

Part 1: Geo-reference Maharashtra Map

Step 1: Open Shapefile

- Open QGIS.

- Add Ind_adm0.shp file.

Step 2: Open Georeferencer

- Go to Raster → Georeferencer.

- Load Maharashtra map image.

Step 3: Add Ground Control Points

- Select points on map image.

- Match them with correct location on Ind_adm0.shp layer.

Step 4: Start Georeferencing

- Select transformation type Linear.

- Click Start Georeferencing.

Part 2: Population Density Analysis

Step 1: Import Raster Files

- Import raster layers:

  - Population Density Grid 1990

  - Population Density Grid 2000

Step 2: Calculate Population Difference

- Go to Raster → Raster Calculator.

- Use expression:

Population_2000 - Population_1990

- Save output raster.

Step 3: Classify Population Change

- Open Layer Properties → Symbology.

- Select Graduated Color.

- Use different colors to represent population increase or decrease.

Part 3: Hillshade Analysis

Step 1: Import Elevation Raster

- Add raster file
10n060e_20101117_gmted_mea300.tif.

Step 2: Extract Area

- Go to Raster → Extraction → Clip Raster by Extent.

- Select North India region.

Step 3: Perform Hillshade

- Go to Raster → Analysis → Hillshade.

- Select elevation raster.

- Generate hillshade output.

Result

Population density change and hillshade analysis of elevation data were successfully performed using QGIS.

✅ Ab Slip 9 & 10 aane wale hain jo thode important practical topics cover karte hain:

- Terrain Analysis

- Natural Calamity CSV analysis

- Attribute table queries

Ye exam me bahut frequently aate hain.

Agar bolo to main Slip 9-10 bhi ready answer format me bana deta hoon (aise hi likhne layak).