Theek hai 👍
Ab Slip 13 aur Slip 14 ko detail + exam scoring format me likh raha hoon. Maine Important Notes / Alert points bhi add kiye hain taaki sir ko lage ki tumhe concept clear hai.

SLIP 13

Aim

To geo-reference the University of Mumbai Fort Campus image using OpenStreetMap and perform digitization of important features in QGIS.

Software / Data Required

- QGIS Software

- Image of University of Mumbai Fort Campus

Part 1: Open QGIS and Add Base Map

Step 1: Open QGIS

- Start QGIS software.

- Create a New Project.

Step 2: Add OpenStreetMap

- Open Browser Panel.

- Expand XYZ Tiles.

- Double-click OpenStreetMap.

The world map will appear on the canvas.

Step 3: Search Fort Campus

- Use Locator Bar / Search Tool.

- Type:

University of Mumbai Fort Campus

- Zoom to the location.

⚠ Important Note (Exam Tip):
OpenStreetMap acts as a reference map for geo-referencing images.

Part 2: Geo-referencing Image

Step 4: Open Georeferencer Tool

- Go to Raster → Georeferencer.

- Click Open Raster.

- Load the Fort Campus image.

Step 5: Add Ground Control Points

- Click Add Point Tool.

- Select a known location on the image.

- Select the same location on OpenStreetMap.

- Repeat the process for 4–6 points.

Examples of points:

- Main gate

- Building corner

- Road intersection

- Landmark

⚠ Important Concept:
Ground Control Points (GCP) help align raster image with real-world coordinates.

Step 6: Set Transformation Parameters

- Click Transformation Settings.

- Select:

Transformation Type → Linear
Resampling Method → Nearest Neighbor

- Select Output raster file location.

Step 7: Start Geo-referencing

- Click Start Georeferencing.

- The image will align correctly with the map.

Part 3: Digitization of Features

Step 8: Create New Shapefile Layers

Create layers for digitization:

| Layer Name | Geometry |
| --- | --- |
| Building | Polygon |
| Road | Line |
| Landmark | Point |

Step 9: Digitize Features

- Enable Toggle Editing Mode.

- Use appropriate tools:

  - Add Polygon Feature → Buildings

  - Add Line Feature → Roads

  - Add Point Feature → Landmarks

- Save edits.

⚠ Exam Note:
Digitization converts map features into vector data layers.

Result

The Fort Campus image was successfully geo-referenced and important features such as buildings, roads, and landmarks were digitized using QGIS.

SLIP 14

Aim

To geo-reference the University of Mumbai Kalina Campus and perform vector digitization of important spatial features using QGIS.

Software / Data Required

- QGIS Software

- Image of University of Mumbai Kalina Campus

Part 1: Add Base Map

Step 1: Open QGIS

- Start QGIS software.

- Create a New Project.

Step 2: Add OpenStreetMap

- Open Browser Panel.

- Select XYZ Tiles → OpenStreetMap.

- Add it to the map canvas.

Step 3: Locate Kalina Campus

- Use Search Tool.

- Type:

University of Mumbai Kalina Campus

- Zoom to the campus area.

Part 2: Geo-referencing Campus Image

Step 4: Open Georeferencer

- Go to Raster → Georeferencer.

- Open the Kalina campus image.

Step 5: Add Ground Control Points

- Select Add Point Tool.

- Mark points on the campus image.

- Select corresponding points on OpenStreetMap.

- Add at least 4–6 points.

⚠ Important Alert for Exam:
More control points provide higher geo-referencing accuracy.

Step 6: Configure Transformation

Set the following parameters:

Transformation Type → Linear
Resampling Method → Nearest Neighbor

Step 7: Start Geo-referencing

Click Start Georeferencing.

The raster image will align with the base map.

Part 3: Digitization of Campus Features

Step 8: Create Vector Layers

Create shapefiles for:

| Layer | Geometry |
| --- | --- |
| Buildings | Polygon |
| Roads | Line |
| Parking | Polygon |
| Bus Stop | Point |

Step 9: Digitize Features

- Enable Editing Mode.

- Digitize the features using tools:

  - Polygon tool → Buildings and Parking

  - Line tool → Roads

  - Point tool → Bus stop.

- Save the edits.

⚠ Important Concept:
Vector digitization allows conversion of satellite imagery into structured GIS datasets.

Result

The Kalina campus image was successfully geo-referenced and vector layers were created to represent campus features such as buildings, roads, parking areas, and bus stops.

✅ Ab Slip 15–16 aane wale hain jisme usually hota hai:

- Spatial Query (Select by Location / Expression)

- Buffer Analysis

- Overlay Analysis

Ye GIS practical me very important aur scoring topics hote hain.

Agar bolo to main Slip 15–16 bhi isi tarah detailed + exam scoring format me abhi bana deta hoon.