Ab main Slip 3 aur Slip 4 ka exam-ready answer de raha hoon. Tum ise direct practical exam me step-by-step likh sakte ho. Maine wording bhi teacher-friendly aur accurate rakhi hai.

SLIP 3

Aim

To create a map based on given assumptions and perform spatial queries using QGIS.

Part 1: Create Map

Step 1: Open QGIS

- Start QGIS software.

- Create a new project.

- Set CRS as WGS84 (EPSG:4326).

Step 2: Create Highway Layer

- Go to Layer → Create Layer → New Shapefile Layer.

- Select Line Geometry.

- Add field:

  - id (Text)

- Save layer as Highway.shp.

- Enable Editing Mode and digitize one highway with id = 375.

Step 3: Create Farms Layer

- Create Polygon Layer called Farm.shp.

- Fields:

  - id (Integer)

  - own_name (Text)

- Digitize 6 farms.

- Ensure 2 farms are near the highway.

Example:

| id | own_name |
| --- | --- |
| 1 | Farm A |
| 2 | Farm B |
| 3 | Farm C |
| 4 | Farm D |
| 5 | Farm E |
| 6 | Farm F |

Step 4: Create Paths Layer

- Create Line Layer named Path.shp.

- Field:

  - id (Integer)

- Digitize 4 narrow paths between farms.

Step 5: Create Main Road

- Create Line Layer named MainRoad.shp.

- Digitize a road passing near farms and touching the highway.

Step 6: Create Farm House Layer

- Create Point Layer called FarmHouse.shp.

- Fields:

  - id

  - frm_id

- Add 2 farm houses inside two farms.

Step 7: Create Residential Areas

- Create Polygon Layer named Residential.shp.

- Add 4 residential areas.

- Ensure:

  - One near farm

  - One near main road.

Step 8: Create Primary School Layer

- Create Point Layer called School.shp.

- Add 2 primary schools.

Step 9: Create Bus Stop and Auto Stand

- Create Point Layer named Transport.shp.

- Fields:

  - id

  - name

- Add:

  - Bus Stop

  - Auto Stand

- Place them near main road.

Step 10: Add Grampanchayat Office

- Create Point Layer called Office.shp.

- Add one Grampanchayat office.

Step 11: Add Wells

- Create Point Layer called Well.shp.

- Add 4 wells:

  - 3 wells in farms

  - 1 well in residential area.

Part 2: Perform Queries

Query 1: Show Farms containing Wells

- Use Vector → Spatial Query.

- Select Farm layer.

- Condition: Contains Well layer.

- Output shows farms having wells.

Query 2: Calculate Farm Area

- Open Farm Attribute Table.

- Click Field Calculator.

- Create field:

  - Name: Area

  - Type: Decimal

- Use expression:

$area

Sort descending to find largest farm.

Query 3: Paths intersecting Highway

- Go to Vector → Geoprocessing Tools → Intersect.

- Input layer: Path

- Overlay layer: Highway.

- Output shows paths intersecting highway.

Result

The map was created successfully and spatial queries were performed to analyze farm areas and intersections using QGIS.